TABLE OF CONTENTS

keyboard.device/CMD_CLEAR

keyboard.device/KBD_ADDRESETHANDLER

keyboard.device/KBD_READEVENT

keyboard.device/KBD_READMATRIX

keyboard.device/KBD_REMRESETHANDLER

keyboard.device/KBD_RESETHANDLERDONE

keyboard.device/CMD_CLEAR                           keyboard.device/CMD_CLEAR

NAME

CMD_CLEAR -- Clear the keyboard input buffer.

FUNCTION

Remove from the input buffer any keys transitions waiting to

satisfy read requests.

IO REQUEST

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Command	CMD_CLEAR

io_Flags	IOB_QUICK set if quick I/O is possible

keyboard.device/KBD_ADDRESETHANDLER       keyboard.device/KBD_ADDRESETHANDLER

NAME

KBD_ADDRESETHANDLER -- Add a keyboard reset handler.

FUNCTION

Add a function to the list of functions called to clean up

before a hard reset generated at the keyboard.  The reset

handler is called as:

ResetHandler(handlerData)

a1

IO REQUEST

io_Message	mn_ReplyPort set

io_Device	preset by OpenDevice

io_Unit		preset by OpenDevice

io_Command	KBD_ADDRESETHANDLER

io_Data		a pointer to an interrupt structure.

is_Data 	the handlerData pointer described above

is_Code	the Handler function address

NOTES

Few of the Amiga keyboard models generate the communication codes

used to implement this reset processing.  Specifically, only the

Euro a1000 (rare), and the B2000 keyboard generate them.

The interrupt structure is kept by the keyboard device until a

RemResetHandler command is satisfied for it, but the

KBD_ADDRESETHANDLER command itself is replied immediately.

keyboard.device/KBD_READEVENT                   keyboard.device/KBD_READEVENT

NAME

KBD_READEVENT -- Return the next keyboard event.

FUNCTION

Read raw keyboard events from the keyboard and put them in the

data area of the iORequest.  If there are no pending keyboard

events, this command will not be satisfied, but if there are

some events, but not as many as can fill IO_LENGTH, the

request will be satisfied with those currently available.

IO REQUEST

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Command	KBD_READEVENT

io_Flags	IOB_QUICK set if quick I/O is possible

io_Length	the size of the io_Data area in bytes: there

are sizeof(inputEvent) bytes per input event.

io_Data		a buffer area to fill with input events.  The

fields of the input event are:

ie_NextEvent

links the events returned

ie_Class

is IECLASS_RAWKEY

ie_Code

contains the next key up/down reports

ie_Qualifier

only the shift and numeric pad bits are set

ie_SubClass, ie_X, ie_Y, ie_TimeStamp

are not used, and set to zero

RESULTS

This function sets the error field in the IORequest, and fills

the IORequest with the next keyboard events (but not partial

events).

keyboard.device/KBD_READMATRIX                 keyboard.device/KBD_READMATRIX

NAME

KBD_READMATRIX -- Read the current keyboard key matrix.

FUNCTION

This function reads the up/down state of every key in the

key matrix.

IO REQUEST INPUT

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Command	KBD_READMATRIX

io_Flags	IOB_QUICK set if quick I/O is possible

io_Length	the size of the io_Data area in bytes: this

must be big enough to hold the key matrix.

io_Data		a buffer area to fill with the key matrix:

an array of bytes whose component bits reflect

each keys state: the state of the key for

keycode n is at bit (n MOD 8) in byte

(n DIV 8) of this matrix.

IO REQUEST OUTPUT

io_Error

IOERR_BADLENGTH - the io_Length was not exactly 13 bytes.

The buffer is unchanged.  This is only returned

by V33/V34 kickstart.

io_Actual	the number of bytes filled in io_Data with key

matrix data, i.e. the minimum of the supplied

length and the internal key matrix size.

NOTE

For V33/V34 Kickstart, io_Length must be set to exactly 13 bytes.

RESULTS

This function sets the error field in the IORequest, and sets

matrix to the current key matrix.

keyboard.device/KBD_REMRESETHANDLER       keyboard.device/KBD_REMRESETHANDLER

NAME

KBD_REMRESETHANDLER -- Remove a keyboard reset handler.

FUNCTION

Remove a function previously added to the list of reset

handler functions with KBD_ADDRESETHANDLER.

IO REQUEST

io_Message	mn_ReplyPort set

io_Device	preset by OpenDevice

io_Unit		preset by OpenDevice

io_Command	KBD_REMRESETHANDLER

io_Data		a pointer to the handler interrupt structure.

keyboard.device/KBD_RESETHANDLERDONE     keyboard.device/KBD_RESETHANDLERDONE

NAME

KBD_RESETHANDLERDONE -- Indicate that reset handling is done.

FUNCTION

Indicate that reset cleanup associated with the handler has

completed.  This command should be issued by all keyboard

reset handlers so that the reset may proceed.

IO REQUEST

io_Message	mn_ReplyPort set

io_Device	preset by OpenDevice

io_Unit		preset by OpenDevice

io_Command	KBD_RESETHANDLERDONE

io_Data		a pointer to the handler interrupt structure.

NOTES

The keyboard processor itself performs the hardware reset, and

will time out and perform the reset even if some reset handlers

have not indicated yet that the reset may proceed.  This

timeout is several seconds.