%N transfix

%D Correct for leakage near transit of DEC ~ -30 sources.

%P rjs

%: uv analysis

%B

This corrects for polarisation leakage caused by errors in

the ATCA antenna orientation. The importance of this effect

goes inversely with the square of the difference of the source

declination from -30. It is most extreme at transit.

To run this task, you must have the pointing parameter file used

at the ATCA during your observation.

To first order, it is not too important whether this correction is

applied before or after gpcal is used, particularly for sources

very close to DEC=-30. Some experimentation might be helpful.

%A vis

The name of the input uv data set. No default.

%A select

The normal uv selection commands. The default is to select everything.

%A line

The normal uv linetype in the form:

line,nchan,start,width,step

The default is all channels. Note that if there are multiple

spectral windows, the fitting process is performed on each window

separately.

%A pparams

Pointing parameter table. These tables are stored on LEON at

Narrabri in the file at$log:pparams.log. It is usually entered as an

indirect parameter file. The table consists of 6 lines (one per

antenna), each consisting of 12 numbers (the first number being the

antenna number).

%A options

Task enrichment options. Possible values are

replace  Replace the XY and YX correlations with an estimate

of the error. This option is only meaningful for

a polarisation-calibrated unpolarised dataset.

Additionally there are the standard options to turn off calibration:

nocal    Do not apply antenna calibration.

nopol    Do not apply leakage correction.

nopass   Do no apply bandpass calibration.

%A out

The name of the output uv data-set. There is no default name.