%N gpedit

%D Edit the gain table.

%P mchw

%: calibration

%B

Gpedit is a MIRIAD task which modifies calibration tables.

See also:  GPLIST

%A vis

The input visibility file, containing the gain file to modify.

No default.

%A select

Normal uv data selection commands. See the help on "select" for

more information. Currently only antenna, time, and amplitude

selection is supported. The amplitude selection applies to the

gains, not the uv-data, and can be used to flag or replace bad

gain amplitudes. The default is to select everything.

%A feeds

The polarisation feeds affected (e.g. R, L, X or Y). Default is

all feeds.

%A gain

This gives the complex-valued gain used in the `multiply' and

`replace' options (see below). It is given as an amplitude

and phase (in degrees). For example gain=2,90 produces a gain

with a amplitude of 2 and phase of 90 degrees. The default is 1,0.

%A options

This gives extra processing options. Several values can be given,

separated by commas. Option values can be abbreviated to uniqueness.

The following options operate on the gains:

replace   The existing gains are replaced by the value

given by the `gain' keyword. NOTE: If no other

options are given, the replace option is performed.

multiply  The existing gains are multiplied by the gain

given by the `gain' keyword.

flag      The existing gains are flagged as bad.

amplitude The phases of the existing gains are set to 0.

phase     The amplitudes of the existing gains are set to 1.

scale     The phase of the gains is multiplied by the factor

given by the `gain' keyword.

dup       Convert a single-polarization gain table into a dual

polarization table.

The following option operates on the polarization leakages:

reflect   The existing leakages are made to possess a

symmetry: For each antenna, we make

DX = -conjg(DY).

zmean     Subtract an offset, so that the mean leakage is 0.

Example:

gpedit vis=cyga gain=14.4,90 select=ant(1,2) feeds=X