%N mosmem

%D Maximum Entropy deconvolution for a mosaiced image

%P rjs

%: deconvolution

%B

MOSMEM is a MIRIAD task that performs a maximum entropy

deconvolution of a mosaiced image.  Optionally it can also

perform a joint deconvolution of a mosaic and single dish image.

MOSMEM will also work correctly on a single-pointing observation

interferometric observation.  In this case, it will be less

efficient than MAXEN, but it could be used when combining single

dish data with a single pointing.

MOSMEM spits out some information as it goes:

RMSFAC is the ratio (actual rms)/(theoretical rms).  It measures

the residuals (i.e. the difference between the dirty image and

the model modified by the point spread function).  RMSFAC should

converge to 1.

NormGrd is normalised gradient in the maximisation process.

Convergence requires this to be less than 0.05

Flux is the sum of all the pixel values in the model.

%A map

One or perhaps two input dirty images (or cubes).  These should

have units of Jy/beam.  The first should be produced by INVERTs

mosaic mode.  The optional second dirty map can be a single-dish

image.  It must be on exactly the same pixel grid as the first

image.  If necessary, use REGRID to make this so.  If two inputs

are given, then a joint deconvolution of the two is performed.

%A beam

One or perhaps two input dirty beams.  The first, corresponding

to the first input dirty map, will be produced by INVERTs mosaic

mode.  There is no default.  The second dirty beam (which must

be given if there are two dirty map inputs) gives the point-

spread function of the single dish dirty map.  This second dirty

beam need not be the same image size as the input dirty maps,

and may be appreciably smaller.  This single-dish beam is

assumed to be position-independent, but it need not be

symmetric.

%A model

An initial estimate of the deconvolved image.  For point

sources, giving a good initial model may help convergence.  In

principle, this only helps convergence, but should not affect

the final solution.  The model could be the output from a

previous run of MOSMEM or any other deconvolution task.  It must

have flux units of Jy/pixel.  The default is a flat estimate,

with the correct flux.

%A default

The default image.  This is the image that the final solution

will tend towards.  The final result will be influenced by this

default if the constrains that the data put on the solution are

weak.  The default is a flat estimate, with the correct flux.

%A out

The name of the output map.  The units of the output will be

Jy/pixel.  It can be input to RESTOR to produce a restored

image, or alternatively to MOSMEM, as a model, to continue the

deconvolution process.

%A niters

The maximum number of iterations.  The default is 30.

%A region

This specifies the region to be deconvolved.  See the User's

Manual for instructions on how to specify this.  The default is

the entire image.

%A measure

The entropy measure to be used, either "gull" (-p*log(p/e)) or

"cornwell" (-log(cosh(p)) -- also called the maximum emptiness

criteria).  Using the maximum emptiness criteria is not

recommended.

%A tol

Tolerance of solution.  There is no need to change this from the

default of 0.01.

%A q

One or two values (corresponding to the mosaic and single dish

observations).  These give estimates of the number of points per

beam.  MOSMEM can usually come up with a good, image-dependent

estimate.

%A rmsfac

MOSMEM must be able to the theoretical rms noise of the input

dirty map(s), and will, by default, attempt to reduce the

residuals to have the same rms as this.  If the true rms noise

is different from the theoretical, you may give the factor to

multiply by to convert from theoretical to true rms noise.

The theoretical rms will usually be an optimistic estimate of

the true noise level.  The true noise will be increased by

calibration errors, confusion, poorly understood distant

sidelobes, etc.  The rmsfac factor gives some "fudge factor"

(usually greater than 1) to scale the theoretical noise estimate

by.  Either one or two values can be given, with the second

value corresponding to the single dish input.

For a mosaic, the theoretical rms is position dependent, and is

determined from information save by INVERT (the mostable table).

For a single dish image, the rms is assumed to be constant

across the field, and given by the "rms" item in the image.  If

the single dish input does not contain this item, then this must

be added before using MOSMEM.  This is easily done: for image

xxxx, use

puthd in=xxxx/rms value=????

where "????" is the rms noise in Jy/beam.

%A factor

The flux calibration factor.  This is only relevant when doing a

joint deconvolution of a mosaic and a single-dish image.  It

gives the factor which the single-dish data should be multiplied

by to convert it to the same flux units as the mosaic.  The

default is 1.  If the "dofactor" options is used (see below),

MOSMEM solves for this parameter.

%A flux

An estimate of the integrated flux of the source.  This

parameter is generally not useful if there is an input single

dish image.  Giving MOSMEM a good value for the integrated flux

will help it find a good solution.  On the other hand, giving a

poor value may do harm.  Normally MOSMEM will NOT constrain the

integrated flux to be this value, but see the "doflux" option

below.  The default is image-dependent for measure=gull, and

zero for measure=cornwell.  A value can be given for each plane

being deconvolved.

%A options

Task enrichment parameters.  Several can be given, separated by

commas.  Minimum match is used.  Possible values are:

doflux     Constrain the solution to have the correct

integrated flux (normally the integrated flux is

not constrained).  The integrated flux is

determined from the "flux" parameter or (if no flux

parameter is given) from the default image.  This

option cannot be used if a single dish input map is

also given.

dofactor   Solve for the flux calibration factor.

verbose    Give lots of messages during the iterations.  The

default is to give a one line message at each

iteration.