%N tvdisp

%D Display an image on a TV device

%P jm

%: visual display

%B

TVDISP is a MIRIAD task to display an image on a TV device.

TVDISP does not clear the screen before it initializes the TV;

see TVINIT for this function.

%A in

This is the standard name for an input dataset -- usually

an image, though sometimes either an image or visibility dataset

or some foreign dataset format. Some tasks support multiple

input datasets at a time (with wildcard supported) whereas

others can handle only a single one at a time. There is

generally no default input name.

%A region

This selects a subregion of an image. Multiple subregions can be

selected, which are "ored" together. The following subcommands are

recognized (each of which can be abbreviated to uniqueness).

images(z1,z2)

Select image planes z1 to z2 inclusive. z2 defaults to z1.

quarter(z1,z2)

Select the inner quarter of the image planes z1 to z2 inclusive.

If both z1 and z2 are missing, then all planes are selected. If

only z2 is omitted, z2 defaults to z1.

boxes(xmin,ymin,xmax,ymax)(z1,z2)

Select the pixels within a box with corners xmin,ymin,xmax,ymax.

z1 and z2 are the same as in the "image" subcommand. If z1 and z2

are omitted, all planes are selected. If the coordinate system

is a relative one, instead of specifying four values for xmin,

ymin, xmax, and ymax, a single value "m" maybe be specified,

which is exactly equivalent to specifying a box of shape

"-m,-m,m,m".

polygon(x0,y0,x1,y1,x2,y2,...)(z1,z2)

Select the pixels within the polygon defined by the list of vertices.

z1 and z2 are the same as in the "image" subcommand. If z1 and z2 are

missing, all planes are selected. If only z2 is omitted, it defaults

to z1.

mask(file)

Select  pixels according to the mask given in the file.

The units of the numbers given in the above commands are, in

general, absolute pixels. But this can be changed (and rechanged)

by using one of the following subcommands.

abspixel

Coordinates are interpreted as absolute pixel values, the default.

relpixel

Coordinates are relative to the reference pixel of the map.

relcenter

Coordinates are relative to the central pixel of the map,

(defined as (naxis1/2+1,naxis2/2+1)).

arcsec

Coordinates are in arcseconds, relative to the reference pixel.

kms

Coordinates in the third dimension are in km/s.

NOTE: This region only permits simple bounding boxes.

The default is to display the entire image.

%A range

The minimum and maximum range used in scaling. The default is

to use the minimum and maximum values of the image.

%A tvchan

The TV channel to display the image on.  This can be 1 to 3 on

the IVAS; 1 to 2 on the MXAS or XMTV servers.  The default is 1.

%A incr

Only every INCR'th pixel is displayed.  This can take from 1

to up to 3 values, giving the increment along the x, y, and z

axis, respectively. The default is chosen such that the selected

region of each image will fit on the screen.

%A tvcorn

This gives the coordinate where the lower left corner of the

image is displayed.  The coordinate system typically runs from

1 to 1024 in x and y, the origin being in the lower left corner.

The default is to center the image on the screen.

%A server

This is used to specify the TV display device. It is giving in the

form:

type@server

or

type@device

The first form causes a connection to be made with a network

display server, whereas the second form uses local hardware

directly. For the network servers, the server process may have

to be started before invoking the Miriad task -- see below.

Currently supported types are:

xmtv          Miriad X server. `server' is the servers hostname.

(use "localhost" if you are not networked)

Normally the xmtv server will be the same host as

the X server. The user will normally invoke the xmtv

server before running any Miriad TV tasks, with

% xmtv

If the X server and xmtv server are on different

hosts, you may need to set the DISPLAY environment

variable on the xmtv server, and use xhost on the

X server.

ivas          IVAS (VMS only). `device' is the VMS device name.

You will require an IIS IVAS for this.

ivserver      IVAS server. `server' is the servers hostname.

Servers are currently installed on

castor.astro.uiuc.edu and bkyast.berkeley.edu.

msss          Miriad Sun Screen server. `server' is the serving

Sun, which can be any Sun with a console. The user

will normally invoke the server before running

any Miriad TV tasks, with

% msss

mxas          Miriad X server. `server' is the servers hostname.

This is an inferior server to xmtv.

There is no default for the server.

%A options

These indicate extra operations that may be performed. One or

more can be given.  Possible values are:

fiddle  Allow interactive modification of colour tables, zoom,

etc.  NOTE: The Fiddle option currently only works

with the IVAS servers!

zoom    Zoom in on the displayed images.  NOTE: Zoom attempts

to display the bounding box of all of the displayed

images.  If more than one channel was loaded, then

the zoomed display will be of the first channel.

movie   Display the sequence of images as a movie.

NOTE: Movie will attempt to zoom onto one image and

then center the display step by step through each

following image (and then return back to the first

image to begin again).  The rate at which the images

will change is a function of the cursor's X position.

The further to the left of the display that the cursor

is located, the faster the rate of change; the further

right, the slower.  NOTE: some displays require a

mouse button press to identify the cursor position.

Functions to control the direction and style of motion

are available if the panel is present.  If not, a

subset of functions will be available to the four

functions A - D (may also be keys F3 - F6).