%N smamfcal

%D Multifrequency antenna and passband calibration.

%P jhz

%: calibration

%B

SmaMfCal is a Miriad task which determines calibration corrections

(antenna gains, delay terms and passband shapes) from a

multi-frequency observation.  The delays and passband are

determined from an average of all the selected data.  The gains

are worked out periodically depending upon the user

selected interval. SmaMfcal implements algorithms for weighting,

continuum vector normalization, and moving smooth prior to solving

for bandpass and gains, which are necessary for handling data

at submillimeter wavelength when the S/N is poor and phase dispersion

is large. The basic solving algorithms are the same as in Mfcal.

%A vis

Input visibility data file. No default. This can (indeed should)

contain multiple channels and spectral windows. The frequency

set-up can vary with time.

%A line

Standard line parameter, with standard defaults.

%A edge

The number of channels, at the edges of each spectral window, that

are to be dropped. Either one or two numbers can be given, being the

number of channels at the start and end of each spectral window to be

dropped. If only one number is given, then this number of channels

is dropped from both the start and end. The default value is 0.

%A select

Standard uv selection. Default is all data.

%A flux

Three numbers, giving the source flux, the reference frequency

(in GHz) and the source spectral index. The flux and spectral index

are at the reference frequency. If not values are given, then SmaMFCAL

checks whether the source is one of its known sources, and uses the

appropriate flux variation with frequency. Otherwise the default flux

is determined so that the rms gain amplitude is 1, and the default

spectral index is 0. The default reference frequency is the mean of

the frequencies in the input data. Also see the `oldflux' option.

%A refant

The reference antenna. Default is 3. The reference antenna needs

to be present throughout the observation. Any solution intervals

where the reference antenna is missing are discarded.

%A minants

The minimum number of antennae that must be present before a

solution is attempted. Default is 2.

%A interval

This gives one or two numbers, both given in minutes, both being

used to determine the extents of the gains calibration solution

interval. The first gives the max length of a solution interval. The

second gives the max gap size in a solution interval. A new solution

interval is started when either the max times length is exceeded, or a

gap larger than the max gap is encountered. The default max length is

5 minutes, and the max gap size is the same as the max length.

%A weight

This gives different ways to determine weights (wt) prior to

solving for bandpass:

-1 -> wt = 1; the same weighting method as used in MFCAL.

1 -> wt ~ amp0**2/var(i); for a normalized channel

visibility, the reduced variance is proportional

to amp0**2/var(i), where amp0 is the amplitude

of the pseudo continuum and var(i) is the variance

of visibility for the ith channel.

2 -> wt ~ amp0**4/var(i)**2;

Default is 2 for SMA and -1 for other telescopes.

if you have stable phase, use -1;

if the phase stability is poor, use 1 or 2;

for a larger planet, 2 is recommended.

For antenna gains' solver:

-1 -> wt = 1; the same weight method that is used in MFCAL.

>0 -> wt = 1/var, where var is the visibility variance.

Defualt is 1/var.

%A options

Extra processing options. Several values can be given, separated by

commas. Minimum match is used. Possible values are:

delay     Attempt to solve for the delay parameters. This can

be a large sink of CPU time.

nopassol  Do not solve for bandpass shape. In this case if a bandpass

table is present in the visibility data-set, then it will

be applied to the data.

interpolate Interpolate (and extrapolate) via a spline fit (to

the real and imaginary parts) bandpass values for

channels with no solution (because of flagging).  If

less than 50% of the channels are unflagged, the

interpolation (extrapolation) is not done and those

channels will not have a bandpass solution

oldflux   This causes SmaMFCAL to use a pre-August 1994 ATCA flux

density scale. See the help on "oldflux" for more

information.

msmooth      Do moving average of the uv data (the real and

imaginary parts) using the keyword smooth parameters

specified prior to solving for bandpass.

opolyfit     Do least-square fit to the bandpass solutions

(the real and imaginary parts) with an orthogonal

polynomial of degree n which can be given in keyword

polyfit.

wrap         Don't unwrap phase while do fit or smooth

the uv data.

averrll      In the case of solving for bandpass of dual

polarizations, averrll gives vector average

of rr and ll bandpass solutions; the mean value

is written into the bandpass table for each of rr

and ll.

%A smooth

This gives three parameters of moving smooth calculation of the

bandpass/gain curves

smooth(1) = K  parameter k giving the length 2k+1 of the averaging

interval; default is 3.

smooth(2) = L  order of the averaging polynomial l; default is 1.

smooth(3) = P  probability P for computing the confidence limits;

default is 0.9.

%A polyfit

polyfit gives a degree of orthogonal polynomial in least-sqaure

fit to the bandpass/gain curves. Default is 3.

polyfit: 1 (linear), 2 (parabolic), 3 (cubic), ....

%A tol

Solution convergence tolerance. Default is 0.001.